HART Railroad

Pittsburgh & Chartiers Valley Division

Scale Operating Instructions  ·  Revision D

Neville Island Operations

Issuing Authority: Office of the Superintendent

Publication EQ-01

Effective 15 July 1999

Revision D

Official Publication

# SECTION 1 — GENERAL RULES

The scale is used to verify and log proper loading of outbound cars prior to departure from South Yard.

## 1.1 Pre-Scale Alignment & Approach

- Bring the train to a complete stop well before the scale approach track.

- Ensure all handbrakes are fully released and train air is completely cut in.

## 1.2 Slack Management (Coupled Tension)

- Begin the move by gently stretching out the entire string of cars.

- The train must be 100% fully stretched (in draft) before the first car reaches the scale track.

- Maintain this constant tension throughout the entire weigh run.

## 1.3 Speed & Braking

- Maintain a strict, uniform speed between 3 and 5 MPH.

- Avoid throttle adjustments, rapid acceleration, or drifting while cars are on the scale.

- Do not use independent, dynamic, or automatic brakes during weighing.

- All scale movements must be performed at restricted speed.

## 1.4 Safety & Operating Notes

- Never leave the Track Scale Terminal unattended while a car is under weigh.

- Protect shove movements at all times.

- Report scale failures or abnormal readings immediately.

# SECTION 2 — SCALE CALIBRATION PROCEDURE

## 2.1 Preparing the Scale

- Verify the scale is completely clear.

- Open the Track Scale Terminal and select Calibrate mode.

## 2.2 Calibration Procedure

- Place the Scale Test Car on the LEFT sensor position and weigh the car.

- Adjust the LEFT sensor until Error reads 0.00 tons, then lock the sensor.

- Repeat for the CENTER and RIGHT sensor positions.

- When all three sensors show zero error and the terminal indicates READY, select Save calibration.

## 2.3 Returning the Scale to Service

- Clear the scale and return the Track Scale Terminal to Weigh mode.

- Return the Scale Test Car to South Yard.

# SECTION 3 — WEIGHING OUTBOUND CARS

## 3.1 Preparing for Weighing

- Open the Track Scale Terminal and select Weigh mode.

- Observe Section 1 movement rules, then spot the car on the scale.

- Select the car from Cars at Scale / Inbound Train.

## 3.2 Track Scale Display

The terminal displays Adjusted Gross Weight and Net Load for the car on the scale.

- PASS — Record the certified net weight. The car is within prescribed tolerance and approved for outbound movement.

- REJECTED — Overload or left/right imbalance beyond prescribed tolerance; do not release the car.

- Assign a reload order for return to the originating industry.

- Rejected cars must not depart South Yard until reweighed and approved.